## EECE 3624: Microprocessors

## Fall 2026

## Dr. Wei

PreLab05

Jose De Los Santos

September 22, 2026

# Prelab Questions

Question 1. State the function of each I/O port register: DDRx, PORTx, and PINx. Then, if DDRD2 = 0 and PORTD2 = 1, explain how PD2 is configured.

Answer1: DDRx: The DDRx controls the direction of the pins in PORTx

PORTx: The PORTx controls the value at the output by controlling the pull-up resistor

PINx: Reads the input at the pins.

If DDRD2= 0 and PORTD2=1, then PD2 is an input with the internal pull up resistor is enabled.

Question 2. Complete the joystick interrupt wiring below, and briefly explain why the external jumper wires are required.

Answer2: Joystick UP: PB3 → PD3 / INT3

Joystick DOWN: PB1 → PD1 / INT1

Why are the jumper wires required? – PORTB.1 and PORTB.3 cannot generate interrupts. The    jumper wires physically connect the joystick to the external-interrupt pins.

Question 3. For external interrupts, state the purpose of EICRA, EIMSK, the SREG Ibit/ SEI, and the interrupt vector. Which one determines whether the interrupt is triggered by a rising or falling edge?

Answer3. EICRA: External Interrupt Control Register A; controls how the external interrupt is triggered

EIMSK: External Interrupt Mask Register; enables/disables specific e. interrupts.

SREG I-bit / SEI: Global Interrupt Enable bit

Interrupt vector: location in memory that tells the processor where to go once an interrupt is    triggered

Rising/falling-edge selection is controlled by: EICRA

Question 4: The ISRs change BlinkFreq while the main program uses BlinkFreq to control the blink rate. Complete the logic below and state what should happen at the two limits.

Answer4. UP ISR: if BlinkFreq  LESS THAN 15, then INCREMENT BlinkFreq

DOWN ISR: if BlinkFreq  GRATER THAN 1, then DECREMENT BlinkFreq

If BlinkFreq = 15 and UP is selected: BlinkFreq does not change, stays at 15

If BlinkFreq = 1 and DOWN is selected: BlinkFreq does not change either, stays at 1

Question5: LED0–LED3 display BlinkFreq in 4-bit binary on PORTC.3:0 (PC3 = MSB,

PC0 = LSB). Write the four output bits for each value.

Answer5.   BlinkFreq = 1: PC3 PC2 PC1 PC0 = 0 0 0 1

BlinkFreq = 5: PC3 PC2 PC1 PC0 =  0 1 0 1

BlinkFreq = 10: PC3 PC2 PC1 PC0 = 1 0 1 0

BlinkFreq = 15: PC3 PC2 PC1 PC0 = 1 1 1 1

Question6. Assume BlinkFreq = 6. The user presses and releases the UP joystick. Number the following events in the order they occur.

1 A rising edge occurs on the interrupt pin.

2 The corresponding ISR begins.

3 BlinkFreq becomes 7.

4 LEDs 0–3 display 0111.

5 RETI returns control to the main program.

6 The main program uses the new blink rate.

Github link:

https://github.com/Jedelo90/EECE3624-Lab05-JoseD.git